College of Engineering and Technology

Department of Computer Engineering

Arab Academy for Science, Technology & Maritime Transport

PROJECT DOCUMENTATION Web Engineering

Project Title: Mobile Encryption System - Secure Voice Communication over LTE

| Supervised By | Dr. Amr Fahmy |
| --- | --- |
| Course | Embedded Systems Design |
| Semester | Spring 2026 (Semester 8) |
| Academic Year | 2025 / 2026 |

# Table of Contents

Table of Contents	2

1.  Project Team	3

1.1  Team Members	3

1.2  Sub-Group Distribution	3

2.  Introduction & Problem Statement	4

2.1  Background	4

2.2  Problem Statement	4

2.3  Motivation	4

3.  Project Objectives	5

4.  Technology Stack	6

5.  System Architecture & Design	7

5.1  High-Level Architecture	7

5.2  Frontend Architecture	7

5.3  Backend Architecture	7

5.4  Database Schema	7

6.  Project Steps & Phases	8

7.  Group Task Distribution	9

8.  Development Timeline	10

9.  Full Project Documentation	11

9.1  Features & Functional Requirements	11

9.2  Non-Functional Requirements	11

9.3  API Documentation	11

9.4  UI/UX Design Notes	11

9.5  Testing Plan	12

9.6  Deployment & DevOps	12

10.  Submission Checklist	13

# 1.  Project Team

## 1.1  Team Members

| Full Name | College ID | Role / Responsibility | Email |
| --- | --- | --- | --- |
| Karim Mohamed Ismail | 221104798 | Hardware + Firmware (main.c) | 221104798@student.aast.edu |
| Rawan Mohamed Ismail | 221006089 | Hardware + Firmware (main.c) | 221006089@student.aast.edu |
| Omar Mohamed Shafei | 221005236 | Encryption (AES-CTR + Relay) | 221005236@student.aast.edu |
| Malak Wael Ghabn | 221004011 | Hardware + Encryption | 221004011@student.aast.edu |

## 1.2  Sub-Group Distribution

Divide the team into sub-groups based on project layers or modules.

| Sub-Group | Member(s) | Module / Layer | Key Deliverable |
| --- | --- | --- | --- |
| Hardware | Karim, Rawan, Malak | Analog audio chain, modem + ESP32 bridge wiring, schematic, BOM, physical assembly | Working two-board prototype, bring-up checklist |
| Firmware | Karim, Rawan | STM32 main.c - call state machine, ADC/DAC, jitter buffer, noise gate, +IPD parser | Flashed firmware, intelligible voice over LTE |
| Encryption | Omar, Malak | AES-128-CTR (crypto.c/.h), pre-shared key, fingerprint, decrypt pump, server-side verification | End-to-end encrypted call with KEYFP MATCH |
| Server / Network | Omar | Python relay (relay.py): REG/CALL/ANS/HUP protocol, byte forwarding, ngrok tunnel, .wav capture | Public relay endpoint + decrypted-audio demo files |

# 2.  Introduction & Problem Statement

## 2.1  Background

Provide context about the domain or field your web application addresses. Explain the current landscape, existing solutions, and why this project is needed.

Voice calls over public mobile networks are widely available but unprotected end-to-end - anyone with access to the carrier or to the relay infrastructure can eavesdrop. Off-the-shelf secure-voice products exist but they are not available as small, buildable, embedded devices a student can wire up. This project closes that gap: two STM32 boards each with an LTE modem, exchanging AES-128-CTR encrypted voice through a thin public relay that never sees plaintext.

## 2.2  Problem Statement

Describe the core problem your web application solves. Be specific and concise.

- Voice calls over LTE are NOT encrypted end-to-end - the carrier and any intermediate relay see plaintext audio.

- There is no small, buildable, embedded reference for an encrypted voice link that a student can assemble and demo.

- Existing secure-voice products are closed source and not extensible for an educational embedded-systems context.

- Open-source embedded crypto walkthroughs typically stop at the algorithm and do not address the real-time, jitter-buffered, interrupt-driven audio path.

## 2.3  Motivation

Explain why solving this problem matters and what impact the solution will have on users or the target audience.

Voice calls are routinely surveilled — carriers, governments, and any compromised relay between the two ends can listen in, and the encryption modern networks use stops at the carrier rather than reaching the user. This project gives ordinary people a small, self-contained device that encrypts speech end-to-end before it ever leaves the microphone, so the relay (and anyone tapping the LTE link) sees only random noise. The same design is directly useful in journalism, healthcare, executive communication, and any field-deployed scenario where the conversation must stay private even when the network in between cannot be trusted.

# 3.  Project Objectives

By the end of this project, the team will have achieved the following measurable goals:

- Build two STM32-based devices that capture voice, encrypt it, and send it over LTE to each other through a public relay.

- Implement AES-128 in CTR mode in C, with a pre-shared key, and a key-fingerprint protocol to detect a mismatch before any call.

- Build a Python relay server (asyncio) that brokers calls and captures both on-the-wire ciphertext and decrypted audio as .wav files for demo proof.

- Run the relay behind a public ngrok TCP tunnel so the two devices can reach each other from any 4G network.

- Achieve intelligible voice quality on the receive side with a <=250 ms one-way buffering delay over the LTE link.

- Verify, with measured evidence (entropy ~ 8.0 bits/byte on the wire, <= 5 bits/byte after AES decryption), that an eavesdropper on the relay sees only random noise.

# 4.  Technology Stack

List every technology, library, and tool used in the project.

| Layer | Technology / Tool | Purpose / Notes |
| --- | --- | --- |
| MCU | STM32F401CCU6 (Cortex-M4F @ 84 MHz, BlackPill) | Main processor on both boards |
| Microphone | MAX9814 (with AGC) into PA0 (ADC1_IN0) | Captures voice (8 kHz, 12-bit) |
| DAC + Amplifier | MCP4725 (I2C) -> LM386 -> headphones | Plays decrypted audio |
| LTE Modem | LilyGO T-A7608 (Cat-4) + ESP32 bridge | Public IP transport over Vodafone Egypt |
| RTOS / Framework | STM32 HAL (bare-metal super-loop) | Peripherals + ISR audio path |
| Toolchain | CubeMX + CMake + arm-none-eabi-gcc (-O2) | Builds Debug at -O2 so AES keeps up in real time |
| Crypto algorithm | AES-128 in CTR mode, pre-shared key | Encrypts every audio packet; key fingerprint at REG |
| Relay server | Python 3.12 + asyncio | Byte-forwards, verifies KEYFP, captures encrypted/decrypted WAVs |
| Public tunnel | ngrok TCP (tcp.eu.ngrok.io:port) | Exposes the relay's port 5555 to the internet |
| Version control | Git + GitHub (Karimkimoz74/Mobile_Encryption_System) | Source + documentation hosting |

# 5.  System Architecture & Design

## 5.1  High-Level Architecture

Describe how the major components of your application interact (client, server, database, external APIs, etc.).

Architecture: each device runs the STM32 firmware (mic -> ADC -> AES-CTR encrypt -> LTE TCP -> relay -> LTE TCP -> AES-CTR decrypt -> DAC -> speaker). The relay is a thin byte-forwarding Python program that never holds plaintext audio. Both devices share the same pre-shared AES key (compiled in via crypto_keys.h).

## 5.2  Frontend Architecture

Describe the component structure, routing strategy, and state management approach.

- Devices: two physical boards (STM32F401 + MAX9814 + MCP4725 + LilyGO T-A7608).

- On-device state machine: ST_BOOTING -> ST_IDLE -> ST_CALLING / ST_RINGING -> ST_IN_CALL.

- Driven by two buttons (PA1 Call / PTT, PB0 Answer / Hang-up) and the relay's RING / INC / GO / PEER_HUP messages.

## 5.3  Backend Architecture

Describe the API design, middleware, and server structure.

- Relay protocol - text lines: REG <phone>, CALL <peer>, ANS, HUP, and the relay-initiated RING / INC <caller> / GO / PEER_HUP / PEER_GONE. After GO the socket flips to transparent byte forwarding.

- Server modules: Session (per-socket state), setup_phase (line parsing), talk_phase (byte forwarding + .wav capture), CallSniffer (deferred AES decryption to .wav).

- Authentication: KEYFP fingerprint comparison at REG. Error handling: ngrok / NAT idle-timeout heartbeat (PING).

## 5.4  Database Schema

Describe the main entities, relationships, and key fields.

There is no database. State lives in RAM: adc_buffer[1024] (DMA-filled mic samples), play_buf[8192] (DAC playback ring), enc_rx_buf[4096] (encrypted RX staging), lte_rx_buf[2048] (line-mode AT responses), and the call_state_t enum.

# 6.  Project Steps & Phases

Outline each development phase with its tasks and expected outputs.

| Phase | Step / Task | Expected Output / Deliverable |
| --- | --- | --- |
| Phase 0-1 | Component sanity, toolchain bring-up (ST-Link, VS Code, CubeMX -> CMake) | LED blink, dev environment proven |
| Phase 2 | Analog talkback - mic -> MAX9814 -> LM386 -> TRRS -> headphones, no MCU | Audible voice through the analog chain |
| Phase 3 | Digital loopback - mic -> ADC -> MCP4725 DAC -> speaker (12-bit, 8 kHz) | Clean local voice loopback |
| Phase 4 | LTE bring-up - AT sequence, attach to Vodafone Egypt, TCP echo to a public test server | End-to-end TCP over 4G verified |
| Phase 5 | Relay server (Python asyncio) + REG/CALL/ANS/HUP/GO protocol, ngrok tunnel, automatic STM32 registration | Public relay reachable, devices auto-register |
| Phase 6 | Two-device full call audio over LTE (plain) - AUTO-VOX, jitter buffer, +IPD parser, NVIC priority fix | Intelligible voice both directions |
| Phase 7 | AES-128-CTR encryption (crypto.c/.h, keygen.py, KEYFP, decrypt_pump) + relay-side decryption capture, build flags raised to -O2 | Encrypted voice end-to-end, KEYFP MATCH, encrypted/decrypted WAVs |
| Phase 8 | Final integration, documentation (HTML walkthroughs, project doc, rubric), demo prep, GitHub push | Submission package and live demo |

# 7.  Group Task Distribution

Assign each feature or module to a team member. Update the Status column as the project progresses.

| Module / Feature | Assigned Member(s) | Sub-Group | Status |
| --- | --- | --- | --- |
| Analog audio chain (mic + amp + DAC + TRRS) | Rawan, Karim, Malak | Hardware | Done |
| LTE modem + ESP32 bridge wiring | Malak, Karim, Rawan | Hardware | Done |
| STM32 firmware - main.c (state machine, audio path) | Karim, Rawan | Firmware | Done |
| ADC + DAC + timer + DMA bring-up (CubeMX) | Karim, Rawan | Firmware | Done |
| LTE AT sequence + +IPD parser | Karim, Rawan | Firmware | Done |
| AES-128 + CTR implementation (crypto.c/.h) | Omar, Malak | Encryption | Done |
| Pre-shared key + keygen.py + KEYFP fingerprint | Omar, Malak | Encryption | Done |
| decrypt_pump + AES integration in main.c | Malak, Omar | Firmware | Done |
| Python relay server + ngrok + WAV capture | Omar | Server | Done |

# 8.  Development Timeline

Adjust weeks to match your actual course schedule.

| Week | Phase / Task | Responsible Sub-Group | Deliverable |
| --- | --- | --- | --- |
| 1-2 | Phase 0-1 - toolchain, blink, sanity | Hardware + Firmware | Dev env |
| 3 | Phase 2 - analog talkback | Hardware | Analog chain |
| 4 | Phase 3 - digital ADC/DAC loopback | Firmware | Local voice |
| 5-6 | Phase 4 - LTE bring-up, TCP echo | Firmware + Server | TCP over 4G |
| 7 | Phase 5 - relay protocol + ngrok | Server | Public relay |
| 8-9 | Phase 6 - full call audio over LTE | All | Voice call |
| 10-11 | Phase 7 - AES-128-CTR encryption + KEYFP | Encryption + All | Encrypted call |
| 12 | Phase 8 - final docs, demo, GitHub push | All | Submission |

# 9.  Full Project Documentation

## 9.1  Features & Functional Requirements

List every feature the application must support.

- Phone-number registration and call setup via the relay (REG / CALL / ANS / HUP protocol).

- End-to-end AES-128-CTR encryption of every audio packet, with split CTR counter halves per direction.

- Real-time audio at 8 kHz / 8-bit, half-duplex AUTO-VOX with a 150 ms jitter buffer and chunk-level noise gate.

- Server-side proof: each call produces an encrypted_*.wav (noise) and decrypted_*.wav (recovered voice) pair.

- Key-mismatch protection: each board announces a DBG KEYFP=... fingerprint at REG; the relay flags a mismatch before any call starts.

## 9.2  Non-Functional Requirements

- Performance - sustained 8 kHz audio in/out, <=250 ms one-way buffering delay, AES decrypt fits inside the modem-send wait windows when built with -O2.

- Security - AES-128 in CTR mode with pre-shared 128-bit key; 32-bit key fingerprint check at REG; no plaintext audio on the wire or at the relay.

- Scalability - relay is single-process asyncio; trivially extendable to many calls in parallel because each session is independent.

- Robustness - automatic socket reconnect on +IPCLOSE, 10 s PING heartbeat against NAT idle timeout, audio fail-counter that triggers a relay reconnect.

- Operability - single button starts a call; second button answers or hangs up; LED blink pattern indicates state.

## 9.3  API Documentation

Document your main API endpoints. Add more rows as needed.

| Message | Direction | Description | Phase Allowed | Reply |
| --- | --- | --- | --- | --- |
| REG <phone> | Device -> Relay | Register a phone number | IDLE | OK / ERR taken |
| CALL <peer> | Device -> Relay | Place a call | REGD | RING / ERR unavail |
| INC <caller> | Relay -> Device | Incoming call notification | REGD | (none) |
| ANS | Device -> Relay | Answer an incoming call | RINGING | GO (to both ends) |

## 9.4  UI/UX Design Notes

Describe the design language, color palette, typography, and key user flows.

- User interface - two push-buttons on each board (Call / Answer).

- Feedback - single status LED: slow blink IDLE, fast blink RINGING/CALLING, solid ON IN_CALL.

- [ Key user flow — e.g., Onboarding → Dashboard → Core Feature ]

No graphical UI - embedded device with physical buttons + LED.

## 9.5  Testing Plan

| Test Type | Tool / Method | Scope | Pass Criteria |
| --- | --- | --- | --- |
| Unit (crypto) | verify_crypto.py + FIPS-197 KAT | AES block + CTR + key fingerprint | AES KAT matches; KEYFP deterministic |
| Integration | Relay log inspection (KEYFP, entropy, [AUD]) | End-to-end registration + key match | [KEY] MATCHES on both boards |
| End-to-end | Live call between two boards via ngrok | Encrypted voice both directions | Both [AUD] = decrypts to audio |
| Audio quality | decrypted_*.wav analysis (Python) | Sample dropouts, click rate, entropy | ~84/s glitches before fix -> ~20/s after |

## 9.6  Deployment & DevOps

- Relay hosting - runs locally on any laptop (Python 3.12); exposed to the internet by ngrok TCP.

- Firmware build - CMake + arm-none-eabi-gcc at -O2 -g3 (Debug preset). Two-step procedure: flip IS_DEVICE_A in device_config.h, build, flash; flip the other way, build, flash.

- Configuration - ngrok host/port are #define'd in main.c; the AES key is the static const in crypto_keys.h, generated by keygen.py.

- Public address - varies per ngrok session, e.g. 5.tcp.eu.ngrok.io:29658.

# 10.  Submission Checklist

☐ System architecture diagram completed ☐ Database ERD finalized and approved ☐ All API endpoints documented (Swagger / Postman collection) ☐ Frontend pages implemented and responsive ☐ Backend API tested and returning correct responses ☐ Authentication and authorization working correctly ☐ Frontend and backend fully integrated ☐ Database seeded with sample data ☐ Unit tests written and passing ☐ End-to-end tests covering critical flows ☐ Application deployed to production URL ☐ Environment variables secured (no secrets in repo) ☐ Code reviewed and merged to main branch ☐ Final project report completed and formatted ☐ Presentation slides prepared ☐ Live demo rehearsed by all team members